Alec Brandt

CS-350

Prof. UIucay

7/19/2026

Milestone 1

Question 1: At what frequency can you see the LED start to blink?

As the PWM frequency is reduced from the initial 60 Hz, the blinking becomes visually noticeable at about 20 Hz to 30 Hz.

Question 2: At what duty cycle is the intensity of the LED perceptibly diminished from the initial 50% duty cycle?

The intensity of the LED is perceptibly diminished from the initial 50% duty cycle when the duty cycle drops below approximately 15% to 10%.

Question 3: When changing the duty cycle of the PWM, the loop used an increment of 5 every tenth of a second. Was this perceptibly smooth? If not, what could you change to improve the visual response? Why?

An increment of 5% every 0.1 seconds was moderately smooth, but some minor "stepping" or "stuttering" in brightness was still perceptible. To improve the visual response and make the fading look completely smooth, two changes can be made:

1. The obvious answer to me is to reduce the step size and delay: Instead of incrementing/decrementing by 5% every 0.1 seconds, we could change the duty cycle by 1% every 0.02 seconds (20ms). This yields a much higher update frequency (50 Hz instead of 10 Hz) with smaller increments, which falls within the range where human visual persistence merges the steps into a single, continuous transition.
2. The less evident approach might be to use a logarithmic or exponential fading curve like I’ve done in animation curves in the past: Since human perception of light is non-linear, a linear increment (0, 5, 10...) feels like it changes very quickly at low duty cycles and very slowly at high duty cycles. Using an exponential scale (e.g., duty = max * (step/total)^2) would match the eye's natural sensitivity and create a more uniform, visually pleasing fade.

Question 4: What function sets the PWM frequency for a GPIO line?

In the RPi.GPIO library, the PWM frequency is set initially when creating the PWM instance using the GPIO.PWM(channel, frequency) constructor. If you need to change the frequency of an existing PWM instance on the fly, you can use the ChangeFrequency(frequency) method (e.g., pwm18.ChangeFrequency(30)).

Question 5: What function sets the duty cycle for a GPIO line?

The duty cycle is initially set and started using the start(duty_cycle) method (e.g., pwm18.start(0)). To update or modify the duty cycle during program execution, the ChangeDutyCycle(duty_cycle) method is called (e.g., pwm18.ChangeDutyCycle(dutyCycle)), where the duty cycle parameter is a number from 0 to 100.