Alec Brandt

CS-350

Prof. Ulucay

7/25/2026

CS 350 Milestone Two: GPIO UART Lab Answers

Question 1: Why do both the SerialTest-Write.py and SerialLightControl-Client.py scripts use the encode() method of the string datatype when writing data to the serial port?

UART serial interfaces transmit raw binary bytes (8-bit values) across physical communication lines. In Python 3, standard text strings are stored in memory as high-level Unicode objects (str). Because hardware serial drivers cannot directly transmit Unicode objects, the encode() method must be called to convert the string into a raw byte stream (bytes). By default, encode() uses UTF-8 encoding, translating each character into its corresponding byte sequence so that PySerial's ser.write() function can send it across the serial port.

Question 2: Why does the SerialTest-Read.py script use the decode() method of the string datatype when reading the data from the serial port?

When reading incoming data from the serial port, ser.readline() fetches raw binary data from the hardware buffer and returns it as a Python bytes object (such as b'on\n'). To process or display this data as human-readable text, decode() converts the incoming UTF-8 byte stream back into a standard Python string (str). Without calling decode(), the output would contain raw byte formatting, and string comparisons like match command: would fail because a bytes object is not equal to a str object.

Question 3: What is the purpose of the try/except block in both the SerialLightControl-Client.py script and the SerialLightControl-Server.py script?

The main purpose of the try/except block is to handle a KeyboardInterrupt exception gracefully when a user stops the script using Ctrl+C. Both scripts run inside infinite loops (while repeat:) to continuously handle serial communication. If a user interrupts the script without exception handling, the program terminates abruptly, which can leave the LED powered on and GPIO pins in an active state. Wrapping the loop in a try/except block catches the interrupt, allowing the script to safely turn off the LED using GPIO.output(18, False) and release the GPIO pins using GPIO.cleanup().

Question 4: Why is it necessary to make sure that the GPIO pins are always returned to their original state at the end of program run?

Resetting GPIO pins to their default state (typically high-impedance input mode) using GPIO.cleanup() is essential for hardware safety and system reliability:

Hardware Protection: Leaving a pin configured as an active output with high voltage creates a risk of damage. If wires are accidentally shorted or disconnected while active, excessive current can damage the Raspberry Pi GPIO controller.

Preventing Pin Conflicts: If a program exits without cleaning up, the operating system keeps the pins registered as in-use or in active states. Subsequent scripts trying to initialize those pins may encounter conflicts, resource locks, or configuration warnings.

Power Efficiency: Leaving output pins driven high continues to consume power and generate heat even after the controlling program has stopped running. Returning pins to a neutral input state prevents unnecessary energy consumption.