Alec Brandt

CS-350

Prof. Ulucay

7/25/2026

Module 4: Display Setup Lab

Question 1: Why do you have a sleep command in your loop?

The `sleep(1)` command pauses loop execution for one second between iterations to prevent the Python `while` loop from consuming 100% of CPU capacity (busy-waiting), which saves power and reduces thermal output on the Raspberry Pi. Additionally, pacing the loop at one-second intervals matches the hardware refresh kinetics of the 16x2 LCD display and synchronizes updates with the system time format (`%H:%M:%S`), preventing screen flickering, visual artifacts, and GPIO bus saturation.

Question 2: What is the purpose of having a text display on an embedded device?

A text display serves as a low-cost, resource-efficient Human-Machine Interface (HMI) and diagnostic tool for embedded devices. It allows "headless" systems—which lack traditional monitors or graphical interfaces—to communicate real-time operational status, system metrics (such as IP address, temperature, or timestamp), and error codes directly to users and technicians without requiring an active network connection or SSH terminal.

Question 3: How can you think of the display device as something that could relate to a state machine?

The display device functions as a visual output observer for an embedded system's Finite State Machine (FSM), where each internal state (e.g., initialization, active monitoring, alert, or error) maps directly to a specific message or layout rendered on the LCD screen during state transitions. Furthermore, the LCD's underlying HD44780 controller operates as an internal hardware state machine, moving sequentially through power-on initialization, command configuration, and data-entry modes as character bytes are received.

Reflection on The Lab

I feel as though my installation went pretty smoothly. I didn’t initially understand the need for the potentiometer until I realized how important contract control on these types of screens were. I also found myself losing track of where I was in the instructions due to the inverted numbers for some of the latter steps in wiring. I ultimately found things got a lot easier when I stopped looking to match breadboard locations and instead reference the actual pin out names like D7 to 26, etc.